### ICP Algorithm Overview

The core of the code is the Iterative Closest Point (ICP) algorithm, which iteratively minimizes the distance between two point clouds (source and target) to align them.  ICP class contains the align method, which is where most work happens.

Steps in the align method:

- Initialization:

  - The rotation matrix R_accum is initialized as an identity matrix (no rotation).

  - The translation vector t_accum is initialized as a zero vector (no translation).

  - R_list, t_list, and corres_list are initialized to store the transformations and correspondences at each iteration for visualization.

- Iterative Process:

  - Correspondence Finding: For each point in the source point cloud (P_current), the algorithm finds the closest point in the target point cloud (Q) based on Euclidean distance. This is done by iterating over all points in the source cloud and finding the point in the target cloud with the minimum distance.

  - Centroid Calculation: The centroids of the matched points are calculated for both source (P_corr) and target (Q_corr).

  - Covariance Matrix & SVD: The algorithm computes a covariance matrix between the centered source and target points. The Singular Value Decomposition (SVD) is used to compute the optimal rotation matrix (R) that aligns the source and target points.

  - Translation Calculation: The translation (t) is computed as the difference between the target centroid and the rotated source centroid.

  - Update Transformation: The accumulated rotation matrix R_accum and translation vector t_accum are updated at each step. These accumulated transformations are used to progressively align the source point cloud to the target.

  - Termination: The process stops if there are too few correspondences (less than 3) or if the maximum number of iterations is reached.

- Transformation Application: At the end of each iteration, the transformation (R, t) is applied to the source point cloud, which is then used as the input for the next iteration.

- Animation: The animate_icp_results_colab function provides a visualization of the alignment process by displaying the source and target point clouds at each iteration, along with their correspondences.

### Complexity

The time complexity of our current ICP algorithm can be analyzed as follows:

- Finding Correspondences:

  - For each point in the source cloud (P_current), we find the closest point in the target cloud (Q). This requires calculating the Euclidean distance between each point in P_current and all points in Q, which is an O(N * M) operation, where N is the number of points in P_current and M is the number of points in Q.

- Singular Value Decomposition (SVD):

  - The SVD step, which is used to compute the rotation matrix, is an O(K^3) operation, where K is the dimensionality of the point cloud (typically 3 in 3D point clouds). Since K = 3, this step is a constant-time operation with respect to the number of points.

- Total Complexity:

  - In total, the complexity of each iteration is dominated by the O(N * M) complexity of finding correspondences.

  - Therefore, the overall time complexity is O(I * N * M), where I is the number of iterations in the ICP process.

### Memory Usage:

- The algorithm stores the transformations (R_list, t_list) and correspondences (corres_list) for each iteration. The memory usage will grow with the number of iterations and correspondences. However, this is relatively manageable unless the point clouds are extremely large.